Date :- 27th March 2019

Wednesday

| Quotation No | :- RW/2018-19/Q_13735 |  |
| --- | --- | --- |
| Concern Person | :- Mr. V N Pathak |  |
| Concern Number | :- | 9674975432 |
| Email Id | :- | dr.v.n.pathak@gmail.com |
| Company Name | :- | Pathak N Pathak |
| Address | :- Country : India |  |
|  |  | State : West Bengal |
|  |  | City : Kolkata |

Quotation For Packaged Drinking Water Project

Dear Sir ,

We are pleased to enclose our detailed Techno-Commercial offer, based on your requirement.

Contents of the Quotation

Commercial Proposal

Terms & Condition

Technical Specification for each product

Machinery Offer

Packaged Drinking Water Project

We hope the same is in line with your requirement. We shall gladly assist you in case you need any further clarification or information.

With warm Regards,

For , Raj Water Technology (Guj.) Pvt. Ltd.

DESHANI KHUSHBU

Designation : Marketing Asst. Manager

Mobile : 7043755777

Commercial Proposal

| Sr | Equipment Name | Qty | Basic Rate | Amount |
| --- | --- | --- | --- | --- |
| No. |  |  |  |  |
| 1 | 5000 Litre Per Hours SS-304 RO Plant With U.V | 1 | 1100000.00 | 1100000.00 |
|  | System, SS Storage Tank 2000 Litre 2-Nos |  |  |  |
| 2 | Drinking Alkaline Ionizer Plant Capacity : 1000LPH | 1 | 1500000.00 | 1500000.00 |
| 3 | Fabricated S.S 304 Storage Tank 2000 Litre | 1 | 150000.00 | 150000.00 |
|  | Thickness 3mm With Transfer Pump |  |  |  |
|  | Automatic Bottle Rinsing, Filling & Capping Machine 60- | 1 | 1400000.00 | 1400000.00 |
|  | BPM (12X12X6) |  |  |  |
| 4 | With Infeed And Out Feed Conveyor Belt,Automatic |  |  |  |
|  | cap elevator |  |  |  |
|  | Semi-Automatic Pet Blow Moulding Machine 2-KVT | 1 | 400000.00 | 400000.00 |
|  | 1-Blow + 1-Conveyor, With Linear Oven & Manual |  |  |  |
|  | Loading Device (Double Cavity Semi Automatic |  |  |  |
|  | Machine) Machine Suitable For Neck Size 46mm, Up to |  |  |  |
| 5 | 1 Litre, 1000 To 1200 BPH, Neck Dia 28mm Double |  |  |  |
|  | Cavity Auto Drop |  |  |  |
| 6 | High Pressure Air Compressor 20-HP | 1 | 250000.00 | 250000.00 |
|  | Water Chiller 3- Ton With 200 Litre Storage Tank | 1 | 130000.00 | 130000.00 |
| 7 |  |  |  |  |
|  | Moisturizer Separator | 1 | 50000.00 | 50000.00 |
| 8 |  |  |  |  |
|  | 1 Litre Mould For 2KVT | 1 | 45000.00 | 45000.00 |
| 9 |  |  |  |  |
|  | Automatic Inkjet Batch Coding Machine | 1 | 200000.00 | 200000.00 |
| 10 |  |  |  |  |
|  | Semi-Automatic Shrink Wrapping Machine For Bulk | 1 | 350000.00 | 350000.00 |
| 11 | Bottle Packing |  |  |  |
| 12 | SS Piping RO Plant To Bottle Filling Machine | 1 | 75000.00 | 75000.00 |
|  |  |  | Total Amount | 56,50,000.00 |

Terms & Condition

- Price

The Quoted Prices Are Ex-factory. (For India)

The Quoted Prices Are FOB. (For Export)

- TAX

GST 18% Extra Applicable. (For India)

- Payment

50% Advance Along With The Purchase Order, 30% After Inspection, & 20% Before Material Dispatch.

In Case Of Cancellation Of Confirmed Order Due To Any Reason, As A Part Of Penalty 25% Amount Of The Order Value Shall Have To Pay By You To Raj Water.

- Packing, Freight & Transit Insurance, & Unloading

Packing Charge Will Be Extra (Depend On Packing Material)

Transport To Be Arranged By The Customer OR If Arranged By Raj Water: Extra At Actual Plus GST.

Transportation Insurance In The Scope Of Buyer.

Unloading In The Scope Of Buyer. Buyer Has To Ready Forklift Or Crane And Unskilled Labor.

- Delivery

Schedule For Dispatch Will Be Notified.

Delivery Dates Are Approximate Only, And Assume Timely Receipt Of All Information, Test Materials & Payment.

- Warranty

We Shall Provide 1 Year Warranty For The Following:

Manufacturing Defects, Raw Water Pump, High Pressure Pump, Treated Water Transfer Pump.

Due To Fluctuation In Power Supply, Attempted Repaired By Person Other Than Our Technical Representative, Improper Handling Or Negligence By End Users, Poor Maintenance, Improper Use Of Equipment, That All Type Of Damage Not Covered In Warranty Create Area.

During The Warranty Period, We Shall Provide Supervisory Services Of Our Technical Representative As And When Required.

- Accommodation

Technician Charge Will Be Extra 1000/- Rs. Per Day.

Travelling, Lodging And Boarding, Local Conveyance And Any Other Miscellaneous Expense Of Our Technical Representative Will Have To Be Borne By Client.

- Validity

Our Offer Is Valid For Your Acceptance For A Period Of 6 Months From The Date; Thereafter It Will Be Subject To Confirmation By Us.

- Image

Images Are For Representation Purpose Only.

- Jurisdiction

All Terms Subject To Rajkot, Gujarat Jurisdiction.

Technical Specification

1. Raw Water Quality

| R.O. plant is | 1500 TDS |
| --- | --- |
| designed based On : |  |
| Hardness : | 300 |

Application :-

2. Treated Water Quality

Suspended Solids :

Nill

Turbidity :

Nill

PH :

7.0 To 7.5

T D S :

Less than 40 ppm

Total Hardness :

30 ppm as CaCo3

Bacteria Coliform :

Nill

Application :-

The above water quality is approximate assumption, We can change property of minerals/ions in water by editing the raw water in R.O. treated water

3. Chlorination Dosing Pump

Type :

Dosing Type

Capacity : :

6 LPH

Make of pumps :

E-dose / I Dose

Power Requirement :	Single phase 240 V

Nos :	1 Nos.

Application :-

To Kill the harmful Bacteria in the raw water as well as To control the biological growth in the raw water, To prevent The organic fouling in the R.O. Membrane elements

4. Chlorination Dosing Tank

Type :

Conical Bottom

Capacity :

100 ltr

M.O.C. :

HDPE

5. Dechlorination Dosing Pump

Type :

Dosing type

Capacity :

6 LPH

Make of pumps :

Make of pumps

Power Requirement :

Single phase 240 V

Application :-

To Kill the harmful Bacteria in the raw water as well as, To control the biological growth in the raw water, To prevent,The organic fouling in the R.O. Membrane elements.

6. Dechlorination Dosing Tank

Type :

Conical Bottom

Capacity : :

100 Ltr

M.O.C :

HDPE

7. Raw Water Feeding Pump

Type :

Centrifugal

Max. Flow :

12.00 m3/HR

Working Flow :

10.00 m3/HR

Working Pressure :

2 Kg. / Cm 2

M.O.C. :

C.I.

Power :

1.5 HP

Manufacturer :

Kirloskar / EQI

Nos. Offered :

1 Nos.

Application :-

To Pump Raw Water at Required Pressure so as to enable Transfer Through The RO Pre-Treatment Stage.

8. Pressure Sand Filter (SS)

Type :

Down Flow Graded Sand Type

Model :

2068/RW/MFG

M.O.C. :

SS 304

Size Dia X Height :

500 mm Dia X 1700 mm

Pipe Line Size :

40 mm.

Sand Qty. :

330 Kgs.

Valves :

SS Valve

Max. Flow :

12.00m3/HR

Working Flow :

10.00 m3/HR

Application :-

By means of Multi size Graded sand filled in the Vessel of stainless steel material, which is natural process of filtration adopted by the nature Raw Water is first filtered by filtration unit in series prior feeding RO Plant. Pressure Sand Filter Unit is provided for removal of suspended matters & turbidity from raw water. PSF Unit is a SS vertical Pressure Vessel. Internally, it is fitted with bottom collecting system. Uniform grades silica quartz sand is charged on the supporting media of pebbles and gravels.

9. Activated Carbon Filter (SS)

Type :

Down Flow Graded Sand Type

Model :

2068/RW/MFG

M.O.C. :

SS 304

Size Dia X Height :

500 mm X 1700 mm

Pipe Line Size :

40 mm

Carbon Qty. :

135Kgs.

Valves :

SS Valve

Max. Flow :

12.00 m3/HR

Working Flow :

10.00 m3/HR

Application :-

To Remove taste, odour, chlorine, colour and. It can absorb contaminated particles of Oil & Grease from the raw water. It will control the BOD & COD in the raw water. Activated Carbon can remove Chlorine, Odour, & Colour other organic material from feed water prior to reverse osmosis. This Carbon granules having larger surface area and specific property to absorb organics.

10. Antiscalant Dosing Pump

Type :

Ele. Diaphragm Type

Capacity :

0 to 6 LPH

M.O.C. :

PP

Dosing Chemical :

Antiscalent

Dosing Rate :

3 to 4 LPH

Make :

E Dose/ I Dose

Nos :

1 Nos

Application :-

The hardness salts of calcium & magnesium are likely to be precipitated if concentration exceeds its solubility limit

- it may faults the membranes resulting into scaling which ultimately leads to poor treated water quality from R.O.System.

11. Antiscaling Dosing Tank

Type :

Conical Bottom

Capacity :

100 Ltr.

M.O.C. :

HDPE

Manufacturer :

Reputed

Nos :

1 Nos

12. Micron Cartridge Filter

Model :

Size :

Micron Rating :

Type of Candle :

Material of Housing :

Nos. of Candles :

Max. Flow :

Working Flow :

MCF 20/RW/SS

20" Long

5 Micron

P. P. Spun / Wound

SS 304

2 Nos. (Jumbo)

12.00m3/HR

10.00 m3/HR

Application :-

To protect the membrane and the high pressure pump from suspended particles by removing them up to 5 Micron.

13. High Pressure Pump

Type :

Vertical Centrifugal Pump

M.O.C. :

SS

Max. Pressure :

13 Kgs. / Cm 2.

Working Pressure :

10 Kgs. / Cm 2.

Max. Flow :

12.00 m3/HR

Working Flow :

10.00 m3/HR

Motor :

7.5 HP

Manufacturer :

2900 RPM

Nos. Offered :

3 Phase

Make of Pump :

PNC / EQI

Application :-

For Reverse Osmosis Process to take place feed pressure of water need to be raised. For this purpose a Vertical Multistage Centrifugal High Pressure Pump in S.S construction is provided R.O. System requires operating pressure of about 14-16 kg/cm^2 . High-pressure pump is fitted with S.S. discharge pipe work with necessary control valves. Necessary instruments like a High Pressure Switch, Pressure Gauge and recirculation valves are provided for safer operation of system

14. RO Membrane

Model :

8040 Element

Type :

TFC Spiral Wound

Material :

Polymide

Size :

8" Dia X 40" Long

Manufacturer :

Toray / Hydranautics / Dow / EQI

Nos. Offered :

5 Nos.

Application :-

To reduce Raw Water T.D.S. through the concept of Reverse Osmosis where R.O. Modules processes Raw Water and produces Purified Water having the Salt Rejection of Raw Water in the range of 90 – 95%. By using Semi permeable Membrane filter elements, where ion filtration process is carried out by applying pressure in the water. So finally we will get Low salt water with low total dissolve solids and High purity. Here will have to drain the water at one side to Wash the Membrane filter elements continuously because of filtration process there is nothing to be polluted in the system.

15. SS Pressure Vessels

| M.O.C. : | SS 304 |
| --- | --- |
| Type : | Horizontal Tube Type |
| Size : | 8" X 80" |
| Max. Pressure : | 400 PSI |
| Working Pressure : | 150 PSI |
| Nos. Offered : | 2 Nos. |

16. Pressure Gauges

| Pressure Gauges : | 7 | Nos. |
| --- | --- | --- |
| Type : | Burdone Tube Type |  |
| Manufacturer : | Raj |  |
| Range : | 0 | - 28 , 0 - 7 Kg/cm2 |

17. Rota Meters

Rota Meters :

Type :

M.O.C. :

Reject Water Range :

Treated Water Range

:

2 Nos.

Float

Acrylic Body

SS Float (0 - 10000 Lph.)

SS Float (0 - 6000 Lph.)

18. Control Panel - PLC Base

Manufacturer :	Astral

19. Pressure Switch

Pressure Switch :

2 Nos.

Type :

Low / High Pr.

Manufacturer :

Denfoss

Model :

IPS 70 / 400

20. Treated Water Storage Tank

Nos. Offered :

1 Nos.

Type :

Vertical Cylindrical Type

M.O.C. :

SS 304

Capacity :

2000 Ltr.

Application :-

To store Reverse Osmosis permeate (product) water. It can remove back pressure in R.O. Membrane elements. It is only storage of the treated Water.

21. Treated Water Transfer Pump

Type :	Monoblock

Manufacturer :	PNC

M.O.C. :	SS

| Capacity : | 0.5 HP |
| --- | --- |

Application :-

To feed water to downstream to micron filters and UV system and filling line

22. Post Micron Cartridge Filter-1

Model :

MCF 20/RW/SS

Size :

20" Long

Micron Rating :

2.0 Micron

Type of Candle :

P. P Spun

Material of Housing :

SS 304

Nos. of Candles :

1 Nos

Working Flow :

5.00 m3/HR

Application :-

During storage, atmospheric or any other contamination may take place in treated water. Hence, after passing water from R.O. System, water is once again passed through a micron cartridge filter. To remove micron particles up to 2.0 Micron sizes.

23. Post Micron Cartridge Filter-2

Model :

MCF 20/RW/SS

Size :

20" Long

Micron Rating :

1.0 Micron

Type of Candle :

P. P. Spun

Material of Housing :

SS 304

Nos. of Candles :

1 Nos

Working Flow :

5.00 m3/HR

Application :-

This Vessel fitted internally with P.P spun micron cartridge elements having absolute ratings of 1.0 micron & 0.2 microns which removes all bacterial impurities present in the water and make the water free of particulates.

24. Post Micron Cartridge Filter-3

Model :

MCF 20/RW/SS

Size :

20" Long

Micron Rating :

0.5 Micron

Type of Candle :

P. P Spun

Material of Housing :

SS 304

Nos. of Candles :

1 Nos

Working Flow :

5.00 m3/HR

Application :-

During storage, atmospheric or any other contamination may take place in treated water. Hence, after passing water from R.O. System, water is once again passed through a micron cartridge filter. To remove micron particles up to 2.0 Micron sizes.

25. U. V. Disinfection System

Type :

Fluorescent Tube

Working Flow :

5000 Lph

Working Pressure :

1 Kg. / Cm 2

Material :

SS 304

Manufacturer :

Raj

Nos. Offered :

1 Nos.

Application :-

For removal of micro biological organisms up to 99.9%.

26. Ozone Generator

Nos. Offered :

1 Nos.

Type :

Natural Air Type

Flow Rate :

5000 Lph.

Ozone Dosing :

1 PPM

Casing :

SS

Manufacturer :

Aqua Zone or Equi

Application :-

Though R.O. System rejects all the micro – organisms like bacterial & virus and even after passing through U.V. System it may redevelop in packed bottles over a period of time. Hence, it is necessary to disinfect RO water prior to bottling filling. So that by using ozone gas generator we are producing ozone gas i.e. O3, & is passed through the water, as ozone is having capacity to kill the bacteria as well as to control the further growth In the stored & packed water. Also it is increasing the level of oxygen in the water, where water Becoming very healthy.

27. Ozone Circulation Pump

Type :	Monoblock

Manufacturer :	PNC

M.O.C. :	SS

| Capacity : | 0.5 HP |
| --- | --- |

28. Ozone Circulation Tank

Nos. Offered :

1 Nos.

Type :

Vertical Cylindrical Type

M.O.C. :

SS 304

Capacity :

2000 Ltr.

Application :-

For required ppm Ozone gas mix up in finish product water. In ozone re-circulating tank water re-circulating by required flow rate pump . Ventury of ozone install out let of Pump it suction ozone and mix-up help with static mixer. This tank is provided with one no. of re-circulating pump and one no. of product water transfer pump to the filling section.

29. Stand For Unit

| Stainless Steel : | S.S 304 |
| --- | --- |

Application :-

Complete stand for unit will be made of S.S.304

30. SS Interconnecting Pipe Lines

| Stainless Steel : | S.S 304 |
| --- | --- |

Application :-

All interconnecting pipelines Raw water Pump to Ozone Circulating tank will be provided of S.S 304 by us